BESPOKE PROPERTY CONCIERGE

Home Safety Assessment

Raw field capture export · SafetyCulture · INTERNAL — not client-facing

| Assessment | Home Safety Assessment (HSA) — Annual |
| --- | --- |
| Audit ID | SC-AUD-000812 (sample / test data) |
| Template | BPC HSA v1 — 18 categories / 165 items |
| Site | Site 8 — 15 Annie Street, Hamilton, QLD 4007 (alpha test) |
| Customer | Craig Greenwood (alpha test) |
| Conducted by | Aaron Holloway (Concierge) |
| Conducted on | 11 June 2026, 9:42am |
| Duration | 1h 58m |
| Items assessed | 126 |
| Action items | 23 (Immediate: 10) |
| Monitor / Not present | 15 monitor · 7 not present |
| Satisfactory | 81 |

This is the internal HSA capture. It is the input to the HSP synthesis pipeline and is never exposed to clients.

Category summary

| Category | Action | Items |
| --- | --- | --- |
| 1. Site & External Grounds | 1 | 9 |
| 2. Roof & Roof Plumbing | 3 | 9 |
| 3. External Walls & Cladding | 1 | 8 |
| 4. Windows & External Doors | 2 | 8 |
| 5. Decks, Balconies & Balustrades | 2 | 7 |
| 6. Driveways, Paths & Paving | 1 | 6 |
| 7. Fencing, Gates & Retaining Walls | 0 | 5 |
| 8. Pool, Spa & Water Features | 1 | 7 |
| 9. Garage & Outbuildings | 0 | 5 |
| 10. Structural & Subfloor | 2 | 7 |
| 11. Internal Walls, Ceilings & Finishes | 0 | 7 |
| 12. Flooring & Floor Coverings | 0 | 5 |
| 13. Kitchen & Joinery | 0 | 7 |
| 14. Bathrooms, Ensuites & Wet Areas | 2 | 7 |
| 15. Plumbing & Hot Water | 1 | 7 |
| 16. Electrical & Lighting | 2 | 7 |
| 17. HVAC & Ventilation | 1 | 7 |
| 18. Safety & Compliance | 4 | 8 |

# 1. Site & External Grounds

| Item | Response | Priority |
| --- | --- | --- |
| Site drainage / surface water flow | Action required | Programmed |
| Stormwater pits & grates | Satisfactory | — |
| Boundary trees / overhang to structures | Monitor | Monitor |
| Garden irrigation operation | Satisfactory | — |
| External tap / hose points | Satisfactory | — |
| Letterbox / entry signage | Satisfactory | — |
| Erosion / soil movement near footings | Satisfactory | — |
| External rubbish / bin store condition | Satisfactory | — |
| Termite management signage / barrier evidence | Not present / N/A | — |

Notes & evidence

Finding — Site drainage / surface water flow: Low point forms against rear patio after rain; recommend regrade / channel drain.

Finding — Boundary trees / overhang to structures: Large eucalypt overhanging roofline NE; arborist review within 6 months.

# 2. Roof & Roof Plumbing

| Item | Response | Priority |
| --- | --- | --- |
| Roof covering — general condition (tile/metal) | Action required | Programmed |
| Box gutters / valleys | Action required | Immediate |
| Eaves gutters & downpipes | Action required | Programmed |
| Flashings & penetrations | Monitor | Monitor |
| Ridge capping / pointing | Satisfactory | — |
| Roof ventilation / whirlybirds | Satisfactory | — |
| Skylights / roof windows | Not present / N/A | — |
| Solar array mounting & cabling | Satisfactory | — |
| Roof space — insulation & sarking | Satisfactory | — |

Notes & evidence

Finding — Roof covering — general condition (tile/metal): Two displaced fasteners at ridge; localised sealant failure. Re-secure and reseal.

Finding — Box gutters / valleys: NE box gutter shows surface corrosion and ponding; clear, treat, monitor for perforation.

Finding — Eaves gutters & downpipes: Debris accumulation in N and E runs; recommend clean and re-secure two brackets.

Finding — Flashings & penetrations: Flue flashing sound but ageing; monitor at next visit.

# 3. External Walls & Cladding

| Item | Response | Priority |
| --- | --- | --- |
| Render / texture coat condition | Action required | Programmed |
| Face brickwork / mortar joints | Satisfactory | — |
| Weatherboard / fibre cement cladding | Not present / N/A | — |
| Expansion / control joints | Satisfactory | — |
| External paint — coverage & adhesion | Monitor | Monitor |
| Sealant to penetrations | Satisfactory | — |
| Sub-floor / wall vents clear | Satisfactory | — |
| Signs of moisture ingress / staining | Satisfactory | — |

Notes & evidence

Finding — Render / texture coat condition: Hairline cracking and efflorescence to south wall; investigate source, patch and repaint.

Finding — External paint — coverage & adhesion: West elevation showing early chalking; repaint likely within 12–18 months.

# 4. Windows & External Doors

| Item | Response | Priority |
| --- | --- | --- |
| Window frames & operation | Satisfactory | — |
| Window seals / weatherstripping | Action required | Programmed |
| External door operation & locks | Satisfactory | — |
| Sliding / stacker door tracks & rollers | Action required | Programmed |
| Flyscreens / security screens | Satisfactory | — |
| Glazing — cracks / seal failure | Satisfactory | — |
| Door / window hardware corrosion | Monitor | Monitor |
| Garage door operation & safety reverse | Satisfactory | — |

Notes & evidence

Finding — Window seals / weatherstripping: Master ensuite window seal perished with water staining to reveal; replace seal.

Finding — Sliding / stacker door tracks & rollers: Living stacker rollers binding; service track and rollers.

Finding — Door / window hardware corrosion: Coastal corrosion beginning on external hardware; monitor.

# 5. Decks, Balconies & Balustrades

| Item | Response | Priority |
| --- | --- | --- |
| Deck structure / framing | Satisfactory | — |
| Decking boards — fixings & rot | Monitor | Monitor |
| Balustrade fixing & rigidity | Action required | Immediate |
| Balustrade gap compliance (<125mm) | Satisfactory | — |
| Waterproofing to elevated/wet decks | Satisfactory | — |
| Deck drainage / fall | Satisfactory | — |
| Coatings / oiling condition | Action required | Programmed |

Notes & evidence

Finding — Decking boards — fixings & rot: Two boards cupping near drip line; monitor and re-fix as needed.

Finding — Balustrade fixing & rigidity: Upper deck balustrade shows movement at two posts; child-gap compliant but re-fix promptly.

Finding — Coatings / oiling condition: Deck oil weathered; recommend clean and re-oil this season.

# 6. Driveways, Paths & Paving

| Item | Response | Priority |
| --- | --- | --- |
| Driveway surface condition | Satisfactory | — |
| Trip hazards / lifting / settlement | Action required | Programmed |
| Paving joints / weed growth | Satisfactory | — |
| Drainage / pooling on paths | Satisfactory | — |
| Steps & external handrails | Satisfactory | — |
| Expansion joints / cracking | Monitor | Monitor |

Notes & evidence

Finding — Trip hazards / lifting / settlement: Path slab lifted ~12mm at side entry; grind or replace to remove trip hazard.

Finding — Expansion joints / cracking: Map cracking to driveway; cosmetic, monitor.

# 7. Fencing, Gates & Retaining Walls

| Item | Response | Priority |
| --- | --- | --- |
| Boundary fencing condition | Satisfactory | — |
| Gates — operation & latching | Satisfactory | — |
| Retaining walls — movement / drainage | Monitor | Monitor |
| Fence footings / posts | Satisfactory | — |
| Automated gate / intercom | Not present / N/A | — |

Notes & evidence

Finding — Retaining walls — movement / drainage: Slight lean to rear timber retaining wall; weep holes clear. Monitor for progression.

# 8. Pool, Spa & Water Features

| Item | Response | Priority |
| --- | --- | --- |
| Pool fence & gate compliance | Action required | Immediate |
| Pool gate gap / climbable zone | Satisfactory | — |
| Pump, filter & plumbing | Satisfactory | — |
| Pool interior / coping / tiling | Monitor | Monitor |
| Water balance / equipment operation | Satisfactory | — |
| Spa / heater operation | Not present / N/A | — |
| Water feature operation & leaks | Satisfactory | — |

Notes & evidence

Finding — Pool fence & gate compliance: Pool gate self-close sluggish; CPR sign faded. Adjust hinge, replace signage for compliance.

Finding — Pool interior / coping / tiling: Minor tile lifting at waterline; monitor.

# 9. Garage & Outbuildings

| Item | Response | Priority |
| --- | --- | --- |
| Garage structure & roof | Satisfactory | — |
| Garage floor — cracking / oil staining | Satisfactory | — |
| Outbuilding / shed condition | Satisfactory | — |
| Storage / shelving safety | Satisfactory | — |
| EV charger installation | Not present / N/A | — |

# 10. Structural & Subfloor

| Item | Response | Priority |
| --- | --- | --- |
| Footings / signs of movement | Satisfactory | — |
| Subfloor bearers & joists | Action required | Immediate |
| Subfloor ventilation | Action required | Programmed |
| Stumps / piers condition | Satisfactory | — |
| Wall / ceiling cracking (structural) | Satisfactory | — |
| Floor level / bounce | Monitor | Monitor |
| Evidence of prior termite activity | Satisfactory | — |

Notes & evidence

Finding — Subfloor bearers & joists: Elevated moisture at bearer grid C3; check ventilation and source, treat timber.

Finding — Subfloor ventilation: Two vents blocked by garden bed; clear to improve cross-flow.

Finding — Floor level / bounce: Slight deflection to rear bedroom floor; monitor.

# 11. Internal Walls, Ceilings & Finishes

| Item | Response | Priority |
| --- | --- | --- |
| Wall finishes — cracking / damage | Satisfactory | — |
| Ceiling — sagging / staining | Monitor | Monitor |
| Cornices & plaster | Satisfactory | — |
| Internal paintwork | Satisfactory | — |
| Internal doors & hardware | Satisfactory | — |
| Skirtings / architraves | Satisfactory | — |
| Staircase / internal balustrade | Satisfactory | — |

Notes & evidence

Finding — Ceiling — sagging / staining: Faint historic stain to laundry ceiling, currently dry; monitor.

# 12. Flooring & Floor Coverings

| Item | Response | Priority |
| --- | --- | --- |
| Timber / engineered flooring | Monitor | Monitor |
| Tiling — cracks / drummy / grout | Satisfactory | — |
| Carpet condition | Satisfactory | — |
| Floor transitions / thresholds | Satisfactory | — |
| Subfloor squeaks / movement | Satisfactory | — |

Notes & evidence

Finding — Timber / engineered flooring: Minor gapping to entry boards (seasonal); monitor.

# 13. Kitchen & Joinery

| Item | Response | Priority |
| --- | --- | --- |
| Cabinetry & hinges | Satisfactory | — |
| Benchtops / splashback | Satisfactory | — |
| Sink, tapware & drainage | Satisfactory | — |
| Appliances — operation (oven/cooktop) | Satisfactory | — |
| Dishwasher — operation & connections | Monitor | Monitor |
| Rangehood / exhaust | Satisfactory | — |
| Soft-close / drawer runners | Satisfactory | — |

Notes & evidence

Finding — Dishwasher — operation & connections: Minor staining under unit, currently dry; monitor seal.

# 14. Bathrooms, Ensuites & Wet Areas

| Item | Response | Priority |
| --- | --- | --- |
| Shower — grout / silicone / waterproofing | Action required | Programmed |
| Bath / basin tapware & drainage | Satisfactory | — |
| Toilet — operation & seals | Satisfactory | — |
| Exhaust fans operation | Action required | Programmed |
| Tiling & falls to waste | Satisfactory | — |
| Vanity / cabinetry water damage | Satisfactory | — |
| Signs of mould / ventilation adequacy | Monitor | Monitor |

Notes & evidence

Finding — Shower — grout / silicone / waterproofing: Main bath shower grout failing with silicone mould; re-grout and reseal to prevent ingress.

Finding — Exhaust fans operation: Ensuite exhaust weak / noisy; service or replace to manage moisture.

Finding — Signs of mould / ventilation adequacy: Early mould spotting to main bath ceiling; improve ventilation.

# 15. Plumbing & Hot Water

| Item | Response | Priority |
| --- | --- | --- |
| Visible water leaks / corrosion | Satisfactory | — |
| Water pressure & flow | Satisfactory | — |
| Hot water system — age & operation | Monitor | Monitor |
| TPR valve / relief & tundish | Action required | Programmed |
| Gas — connections & compliance | Satisfactory | — |
| Stop tap / isolation valves operation | Satisfactory | — |
| Drainage — gully / overflow | Satisfactory | — |

Notes & evidence

Finding — Hot water system — age & operation: HWS operating; approx 9 years old, plan for replacement in cycle.

Finding — TPR valve / relief & tundish: No recent TPR discharge evidence and no tundish observed; plumber to verify.

# 16. Electrical & Lighting

| Item | Response | Priority |
| --- | --- | --- |
| Main switchboard — condition & labelling | Action required | Immediate |
| RCD / safety switch operation | Action required | Immediate |
| Power points & switches | Satisfactory | — |
| Internal & external lighting | Monitor | Monitor |
| Exposed / damaged wiring | Satisfactory | — |
| Switchboard surge / mains condition | Satisfactory | — |
| Data / comms / network cabinet | Satisfactory | — |

Notes & evidence

Finding — Main switchboard — condition & labelling: Two circuits without RCD protection; licensed electrician to upgrade for compliance.

Finding — RCD / safety switch operation: RCD test not completed on all circuits; full test required.

Finding — Internal & external lighting: Two external fittings with water marking; monitor / replace.

# 17. HVAC & Ventilation

| Item | Response | Priority |
| --- | --- | --- |
| Air conditioning — operation (heat/cool) | Satisfactory | — |
| Filters — condition | Action required | Programmed |
| Condensate drainage | Satisfactory | — |
| Outdoor condenser units condition | Satisfactory | — |
| Ductwork / vents | Satisfactory | — |
| Ceiling / exhaust ventilation general | Satisfactory | — |
| Ceiling fans operation | Satisfactory | — |

Notes & evidence

Finding — Filters — condition: Ducted return filter heavily fouled, restricting flow; clean / replace and set service cadence.

# 18. Safety & Compliance

| Item | Response | Priority |
| --- | --- | --- |
| Smoke alarms — type, age & interconnection | Action required | Immediate |
| Smoke alarm placement / coverage | Action required | Immediate |
| Pool fencing compliance (cross-ref §8) | Action required | Immediate |
| RCD protection (cross-ref §16) | Action required | Immediate |
| Fire / emergency egress clear | Satisfactory | — |
| Handrails / balustrade compliance | Satisfactory | — |
| CO / gas safety (if applicable) | Not present / N/A | — |
| Security — alarm / cameras operation | Satisfactory | — |

Notes & evidence

Finding — Smoke alarms — type, age & interconnection: Hallway unit non-interconnected, dated 2014; upgrade to interconnected photoelectric per QLD legislation.

Finding — Smoke alarm placement / coverage: Coverage does not meet current QLD bedroom/hallway requirement; remediate.

Finding — Pool fencing compliance (cross-ref §8): See §8 — gate self-close and signage non-compliant.

Finding — RCD protection (cross-ref §16): See §16 — circuits without RCD.